MMS + GPRS Configuration Mobistar

- Go to Start/ Settings/ Connections. You will get the following screen.

- In My ISP. You have you normal gprs connection. If it does not exist here are the configuration parameters: Add a new modem connection, choose a name and choose cellular line (GPRS)

Click next

APN: internet.be

Click next

Leave all fields blank

Click on Advanced

Use server-assigned IP address

Click on the Servers tab

Skip this step, use server assigned.

Click Ok and then on Finish

You GPRS settings are done

3. To configure the MMS settings

In My Work Network click on Add a new Modem Connection.:

Choose a new name (e.g. mms mobistar).

Choose a modem: Cellular Line (GPRS) and click next

For the access point name put : mms.be for postpaid

Click next

Put as username: mobistar

As password: mobistar

Leave domain blank

Click on Advanced

Leave the choice on Use Server-Assigned Ip address.

Do not fill the Ip address  as on the picture.

Choose the Servers tab

Choose Use specific server address

Click Ok and then finish

Your GPRS settings for the mms settings are done.

4. Go to Start/programsMMS Composer to fill in the MMS preferences.

Click on Tools/Options

Click on the Servers tab and by click and stay on the screen you’ll get the menu new

You get the following screen:

Choose a server name (e.g. mms mobistar)

In gateway fill: 212.65.63.143

In server address fill in: http://mmsc.mobistar.be

Click ok

Go bavck to the preferences tab

In connect via choose: My Work Network

In Connect to server choose: mms mobistar

Leave the rest as is

Click ok

You are now setup to send and receive mms.

Don’t forget to activate the service  by calling 5655.